5G NR RRC STATE MACHINE

Energy Optimization Research

Full Research Plan, Simulation Architecture & Paper Roadmap

Tools: ns-3 (5G-LENA) | Python | MATLAB | IEEE Paper Structure

# 1. Research Overview & Novel Contribution

This document provides a complete roadmap to simulate, analyze, and publish research on 5G NR RRC state machine optimization. Your work builds on standard 3GPP Release 16/17 RRC behavior and extends it with an adaptive, traffic-aware state transition algorithm designed to minimize UE energy consumption without sacrificing Quality of Service (QoS).

## 1.1 The Core Research Problem

In 5G NR, the UE operates in three RRC states:

| State | Power Level | Connection Status | Primary Issue |
| --- | --- | --- | --- |
| RRC_IDLE | ~5 mW | No connection; paging monitored | High latency to re-connect |
| RRC_INACTIVE | ~15 mW | Context stored at gNB | New in 5G NR — underutilized |
| RRC_CONNECTED | ~800–1500 mW | Full active connection | Energy-expensive if idle traffic |

The problem: Existing 3GPP timer-based transitions (inactivity timer, T310, T311) are static. They do not adapt to traffic patterns, mobility conditions, or application requirements, leading to:

- Unnecessary time spent in RRC_CONNECTED during idle micro-bursts (energy waste)

- Premature transitions to RRC_IDLE causing high re-establishment latency

- RRC_INACTIVE state under-exploited — most implementations ignore it

- Battery drain up to 30–40% higher than theoretically optimal

## 1.2 Your Novel Contribution

Your paper will propose an Adaptive RRC State Transition Algorithm (ARSTA) with the following innovations:

- Traffic Prediction Module: Uses a lightweight exponential weighted moving average (EWMA) on inter-packet arrival times to predict UE inactivity and pre-trigger INACTIVE transitions

- Intelligent INACTIVE State Exploitation: Extends the RRC_INACTIVE dwell time using predicted application idle intervals, drastically cutting CONNECTED duration

- Mobility-Aware Paging Optimization: Adjusts paging area size and DRX cycle length based on UE velocity — slower UEs get longer DRX cycles (more sleep = less energy)

- Handover-Aware State Locking: Prevents state transitions during predicted handover windows to avoid signaling storms

## 1.3 Expected Research Findings

Compared to the standard 3GPP timer-based approach (your baseline), ARSTA is expected to demonstrate:

| Metric | Standard 3GPP | ARSTA (Expected) | Improvement |
| --- | --- | --- | --- |
| UE Energy Consumption | Baseline | ~25–35% lower | Significant |
| State Transition Latency | Baseline | ~10–15% lower | Moderate |
| Unnecessary Transitions | Baseline | ~40–50% fewer | High |
| Paging Success Rate | Baseline | ≥ 98% maintained | Preserved |
| Handover Success Rate | Baseline | Equal or better | Preserved |

# 2. Full Simulation Architecture

## 2.1 Technology Stack

| Tool | Role | Specific Use |
| --- | --- | --- |
| ns-3 + 5G-LENA | Core network simulator | RRC state machine, gNB, UE mobility, handover, paging |
| Python 3.x | Analysis & ML pipeline | EWMA traffic predictor, result parsing, plotting, stats |
| NumPy / Pandas | Data processing | Simulation trace parsing, energy calculation |
| Matplotlib / Seaborn | Visualization | Energy curves, CDF plots, state dwell histograms |
| MATLAB | Validation & modeling | Analytical energy model, cross-validate ns-3 results |
| Bash Scripts | Automation | Batch simulation runs, parameter sweeps |

## 2.2 ns-3 Module: 5G-LENA (nr module)

You must use the 5G-LENA NR module for ns-3, which provides full 5G NR stack simulation including RRC. Install it as follows:

# 1. Clone ns-3 and the NR module

git clone https://gitlab.com/nsnam/ns-3-dev.git

cd ns-3-dev

git clone https://gitlab.com/cttc-lena/nr.git contrib/nr

# 2. Configure and build

./ns3 configure --enable-examples --enable-tests

./ns3 build

# 3. Verify NR module is loaded

./ns3 run scratch/5g-rrc-simulation --PrintHelp

## 2.3 Simulation Scenario Design

Your simulation will model an urban macro-cell deployment with the following parameters:

| Parameter | Value | Justification |
| --- | --- | --- |
| Number of gNBs | 3 (triangular) | Enables handover testing |
| Number of UEs | 20–50 | Varied for scalability analysis |
| Carrier Frequency | 3.5 GHz (FR1) | Common 5G NR sub-6 GHz band |
| Bandwidth | 40 MHz |
| Cell Radius | 500 m |
| UE Mobility Model | RandomWaypointMobility | Simulates real pedestrian/vehicular |
| UE Speed Range | 0–30 m/s (pedestrian to vehicular) | Tests mobility-aware DRX |
| Traffic Model | Bursty Poisson (FTP + VoIP) | Realistic IoT/mobile traffic |
| Simulation Duration | 300 seconds per run | Statistically stable results |
| Repetitions per Config | 10 (different seeds) | Confidence interval generation |
| Inactivity Timer (Baseline) | 10–30 seconds (sweep) |
| DRX Cycle (Baseline) | 1.28 s (long) / 320 ms (short) |

# 3. Phase-by-Phase Implementation Roadmap

## Phase 1: Baseline Simulation (Weeks 1–2)

Goal: Reproduce standard 3GPP RRC behavior in ns-3, establish baseline metrics.

- Set up ns-3 + 5G-LENA. Verify build with example scripts.

- Write the baseline scenario script (C++) implementing standard inactivity-timer RRC.

- Enable RRC state logging via ns-3 trace sources.

- Collect: energy per state (model), transition count, latency, paging events.

- Run with 10 seeds, parse results with Python, compute means + 95% CI.

### Key ns-3 Trace Sources to Enable

// In your C++ simulation script

Config::Connect("/NodeList/*/DeviceList/*/LteUeRrc/StateTransition",

MakeCallback(&RrcStateChangeCallback));

Config::Connect("/NodeList/*/DeviceList/*/LteUeRrc/HandoverStart",

MakeCallback(&HandoverCallback));

// Enable PCAP for packet-level analysis

nrHelper->EnableTraces();

## Phase 2: Energy Model Integration (Week 3)

ns-3 does not have a built-in 5G NR UE energy model — you will implement it in Python using the state dwell times exported from ns-3.

### UE Power Consumption Model (Based on 3GPP TR 38.840)

# python/energy_model.py

POWER_MW = {

'RRC_IDLE':      5.0,   # mW — paging DRX sleep

'RRC_INACTIVE':  15.0,  # mW — context stored, RNA paging

'RRC_CONNECTED': 900.0, # mW — active TX/RX (base)

'TRANSITION':    250.0, # mW — signaling overhead

}

def compute_energy(state_log_df):

energy_mJ = 0.0

for _, row in state_log_df.iterrows():

energy_mJ += POWER_MW[row['state']] * row['dwell_ms'] / 1000.0

return energy_mJ

## Phase 3: ARSTA Algorithm — Python Prototype (Week 4)

Before modifying ns-3 (C++), prototype the adaptive algorithm in Python using replayed ns-3 traces. This lets you iterate quickly.

# python/arsta.py — Adaptive RRC State Transition Algorithm

import numpy as np

class ARSTA:

def __init__(self, alpha=0.3, idle_thresh_s=2.0):

self.alpha = alpha          # EWMA smoothing factor

self.ewma_iat = 0.1         # Initial inter-arrival time estimate (s)

self.idle_thresh = idle_thresh_s

self.velocity_history = []

def update_traffic(self, inter_arrival_s):

self.ewma_iat = self.alpha * inter_arrival_s + (1 - self.alpha) * self.ewma_iat

def predict_idle_duration(self):

# Predicted inactivity window = 3x smoothed IAT

return 3.0 * self.ewma_iat

def recommend_state(self, current_state, ue_velocity_mps):

predicted_idle = self.predict_idle_duration()

# Slow UEs: aggressively transition to INACTIVE

if ue_velocity_mps < 3.0 and predicted_idle > 1.5:

return 'RRC_INACTIVE'

elif predicted_idle > self.idle_thresh:

return 'RRC_IDLE'

return 'RRC_CONNECTED'

## Phase 4: ns-3 C++ Integration (Weeks 5–6)

Implement ARSTA inside ns-3 by modifying the LteUeRrc class. The key modification is overriding the inactivity timer callback:

// nr/src/nr-ue-rrc.cc — override inactivity timer

void NrUeRrc::DoNotifyOutOfSync() {

// Standard: always transition to IDLE

// ARSTA: check predicted idle duration first

double predictedIdle = m_arsta.PredictIdleDuration();

double ueVelocity = m_mobility->GetVelocity().GetLength();

if (ueVelocity < 3.0 && predictedIdle > 1.5) {

SwitchToState(INACTIVE);  // Exploit INACTIVE

} else if (predictedIdle > m_inactivityThreshold) {

SwitchToState(IDLE);

}

// Else: stay CONNECTED

}

## Phase 5: Full Experiment & Analysis (Weeks 7–8)

Run a parameter sweep covering all key variables and collect all metrics needed for the research paper comparison tables.

| Experiment | Parameter Varied | Purpose |
| --- | --- | --- |
| EXP-1 | Inactivity timer: 5, 10, 20, 30s | Sensitivity analysis vs ARSTA |
| EXP-2 | UE count: 10, 20, 30, 50 | Scalability + paging load |
| EXP-3 | UE speed: 0, 3, 10, 30 m/s | Mobility-aware DRX tuning |
| EXP-4 | Traffic rate: low/medium/high burst | Traffic-aware ARSTA response |
| EXP-5 | EWMA alpha: 0.1, 0.3, 0.5, 0.7 | Algorithm parameter tuning |

# 4. Parameters to Measure & Compare with Literature

The following table lists every metric you will collect, how to calculate it, and which existing papers you will compare against. This forms the core of your Results section.

| Metric | Unit | How to Compute | Reference Papers |
| --- | --- | --- | --- |
| UE Energy Consumption | mJ / session | Power model × state dwell time | Aziz et al. (2022), Wang et al. (2021) |
| RRC_INACTIVE Utilization | % of total time | INACTIVE dwell / total sim time | 3GPP TR 38.840, Bhattarai et al. (2023) |
| State Transition Latency | ms | Time between state change events | Islam et al. (2020) |
| Handover Latency | ms | HO start to HO complete timestamps | Zhang et al. (2022) |
| Paging Success Rate | % (0–100) | Paged UEs reached / total paged | 3GPP TS 38.304 |
| Unnecessary Transitions | count / UE / hour | Transitions followed by <2s traffic gap | Kim et al. (2021) |
| Average DRX Sleep Ratio | % | Sleep time / total INACTIVE time | Fowler et al. (2023) |
| Throughput | Mbps | Total bits / simulation time | Standard ns-3 FlowMonitor |
| Re-establishment Delay | ms | IDLE-to-CONNECTED connection time | Liu et al. (2022) |
| Signaling Overhead | msg/UE/hr | Count RRC Setup/Reconfig messages | Dang et al. (2020) |

## 4.1 Python Results Pipeline

# python/parse_results.py

import pandas as pd, numpy as np, matplotlib.pyplot as plt

def parse_rrc_log(filepath):

df = pd.read_csv(filepath,

names=['time_s','node_id','old_state','new_state'])

df['dwell_ms'] = df['time_s'].diff().shift(-1) * 1000

return df

def compute_energy_mj(df):

POWER = {'IDLE':5,'INACTIVE':15,'CONNECTED':900,'TRANS':250}

df['power_mw'] = df['new_state'].map(POWER)

return (df['power_mw'] * df['dwell_ms'] / 1000).sum()

def plot_state_distribution(df, label):

state_time = df.groupby('new_state')['dwell_ms'].sum()

state_time.plot(kind='bar', color=['#2196F3','#4CAF50','#F44336'])

plt.title(f'RRC State Time Distribution — {label}')

plt.ylabel('Total Time (ms)'); plt.tight_layout()

plt.savefig(f'results/state_dist_{label}.pdf', dpi=300)

## 4.2 MATLAB Analytical Validation

% matlab/energy_analytical.m

% Analytical Markov chain model for RRC state machine

% States: 1=IDLE, 2=INACTIVE, 3=CONNECTED

lambda_a = 0.5;  % Arrival rate (packets/s)

mu_i = 0.1;      % IDLE->INACTIVE transition rate

mu_c = 0.3;      % INACTIVE->CONNECTED rate

mu_d = 0.2;      % CONNECTED->INACTIVE rate (ARSTA)

% Transition rate matrix Q

Q = [-mu_i,  mu_i,  0;

0,    -mu_c,  mu_c;

0,     mu_d, -mu_d];

% Solve for steady-state probabilities

pi = solve_steady_state(Q);  % pi = [p_IDLE, p_INACTIVE, p_CONNECTED]

P_mW = [5, 15, 900];

E_avg = dot(pi, P_mW);  % Average power in mW

fprintf('Analytical average power: %.2f mW\n', E_avg);

# 5. Research Paper Structure (IEEE Format)

Target conferences/journals: IEEE WCNC, IEEE VTC, Wireless Communications and Mobile Computing (WCMC), or IEEE Access (open access, faster publication).

## Paper Title (Suggested)

"ARSTA: An Adaptive RRC State Transition Algorithm for Energy-Efficient 5G NR User Equipment"

## Section I: Abstract (~150 words)

- Problem: Static timer-based RRC transitions waste UE energy

- Method: ARSTA — traffic-aware, mobility-aware state management

- Setup: ns-3 5G-LENA simulation, urban macro-cell, 20–50 UEs

- Key result (placeholder): XX% energy reduction, <1% paging degradation

- Significance: Enables longer battery life for 5G IoT/mobile devices

## Section II: Introduction

- Hook: 5G device proliferation, battery life still a bottleneck

- Problem statement: RRC state transitions — the hidden energy drain

- 5G NR introduces RRC_INACTIVE — currently underexploited

- Contributions: List 4 specific, numbered contributions

- Paper organization paragraph

## Section III: Background & Related Work

- 3A. 5G NR RRC State Machine: Describe IDLE/INACTIVE/CONNECTED per 3GPP TS 38.331. Show state transition diagram (draw in draw.io or MATLAB).

- 3B. Existing Energy Optimization Work: Summarize 6–8 papers in a comparison table: what they proposed, what they measured, what they did not address (gap your work fills).

- 3C. Limitations of Prior Work: No work combines traffic prediction + mobility + INACTIVE exploitation together — this is your gap.

## Section IV: System Model

- Network topology: gNB placement, UE distribution

- Channel model: 3GPP UMa path loss

- Traffic model: Poisson bursty arrivals, packet size distribution

- UE energy model: equation with state powers (cite 3GPP TR 38.840)

- Mobility model: Random Waypoint, velocity distribution

## Section V: Proposed Algorithm (ARSTA)

- Algorithm overview and block diagram

- Traffic prediction module — EWMA derivation

- State transition decision logic (pseudocode)

- Mobility-aware DRX tuning equations

- Handover-aware state locking rule

- Complexity analysis: O(1) per decision — lightweight for UE implementation

## Section VI: Simulation Setup

Provide full reproducibility details:

- ns-3 version, NR module version, commit hash

- All simulation parameters (refer to table in Section 2.3 of this document)

- Baseline: standard 3GPP timer-based approach

- Compared schemes: at least 2 prior algorithms from literature

- Statistical methodology: 10 runs per config, 95% confidence intervals

## Section VII: Results & Discussion

- Fig 1: Energy consumption CDF — Baseline vs ARSTA vs [Prior Work 1] vs [Prior Work 2]

- Fig 2: State dwell time distribution (bar chart) — all schemes

- Fig 3: Energy vs UE velocity — shows mobility-awareness benefit

- Fig 4: Energy vs traffic load — shows traffic-prediction benefit

- Fig 5: Paging success rate — confirm ARSTA doesn't degrade reachability

- Fig 6: Handover latency CDF — confirm no HO degradation

- Table II: Full parameter comparison: your work vs 5+ reference papers

## Section VIII: Conclusion

- Re-state contributions achieved

- Quantify improvements (fill in after simulation)

- Limitations: assumes accurate velocity estimation; centralized vs distributed

- Future work: ML-based (LSTM) traffic prediction, O-RAN xApp implementation

# 6. Literature Comparison Table (for Paper Section III)

Use this as a starting framework. Search Google Scholar for actual values from these papers to fill in your Table II.

| Paper | Year | Method | Energy Reduction | Simulator | Gap vs Your Work |
| --- | --- | --- | --- | --- | --- |
| Aziz et al. | 2022 | DRX optimization | ~18% | MATLAB | No INACTIVE, no traffic prediction |
| Kim et al. | 2021 | Timer ML tuning | ~22% | ns-3 LTE | LTE only, no 5G INACTIVE |
| Bhattarai et al. | 2023 | INACTIVE state study | ~15% | Analytical | No mobility adaptation |
| Wang et al. | 2021 | Prediction-based HO | N/A | ns-3 | HO only, no energy model |
| Zhang et al. | 2022 | 5G NR DRX | ~20% | Simulation | No paging optimization |
| Your Work | 2025 | ARSTA (all-in-one) | TBD from sim | ns-3 5G-LENA | Combines all — novel |

Note: The paper references above are representative placeholders. Use Google Scholar with queries like: "5G NR RRC INACTIVE energy optimization", "5G DRX energy efficiency ns-3", "RRC state machine energy 5G NR" to find actual papers for your related work section.

# 7. Recommended Project File Structure

5g-rrc-research/

├── ns3/

│   ├── scratch/

│   │   ├── 5g-rrc-baseline.cc        # Standard 3GPP scenario

│   │   ├── 5g-rrc-arsta.cc           # ARSTA implementation

│   │   └── 5g-rrc-sweep.cc           # Parameter sweep wrapper

│   └── contrib/nr/                   # 5G-LENA module

│       └── model/nr-ue-rrc.cc        # Modified for ARSTA

├── python/

│   ├── energy_model.py               # UE power consumption model

│   ├── arsta.py                      # Algorithm prototype

│   ├── parse_results.py              # ns-3 trace parser

│   ├── plot_results.py               # All figures

│   ├── stats.py                      # CI, hypothesis tests

│   └── run_experiments.py            # Batch experiment runner

├── matlab/

│   ├── energy_analytical.m           # Markov chain model

│   └── validate_ns3.m                # Cross-validation

├── results/

│   ├── raw/                          # ns-3 CSV trace outputs

│   ├── processed/                    # Parsed Pandas DataFrames

│   └── figures/                      # PDF/PNG plots for paper

├── paper/

│   ├── main.tex                      # IEEE LaTeX template

│   ├── references.bib

│   └── figures/

└── scripts/

├── run_all.sh                    # Full experiment automation

└── install_deps.sh               # ns-3 + Python setup

# 8. Recommended 10-Week Timeline

| Week | Phase | Tasks | Deliverable |
| --- | --- | --- | --- |
| 1 | Setup | Install ns-3 + 5G-LENA, verify build, study example scripts | Working ns-3 5G setup |
| 2 | Baseline | Implement baseline C++ script, enable RRC tracing | Baseline simulation running |
| 3 | Energy Model | Python energy calculator, parse ns-3 traces, plot state distributions | Energy baseline numbers |
| 4 | Algorithm Prototype | Python ARSTA implementation, validate on replayed traces | ARSTA prototype validated |
| 5 | ns-3 Integration | Implement ARSTA in C++ (nr-ue-rrc.cc) | ARSTA in ns-3 |
| 6 | ARSTA Experiments | Run EXP-1 to EXP-5, collect all metrics | Raw results dataset |
| 7 | MATLAB Validation | Markov chain model, cross-validate with ns-3 | Analytical validation |
| 8 | Analysis & Plots | Full Python analysis, all paper figures, statistical tests | All 6 figures complete |
| 9 | Paper Writing | Write all sections, fill in results, related work survey | Full paper draft |
| 10 | Review & Submit | Proofread, format IEEE template, submit to target venue | Submitted paper |

# 9. Your Immediate Next Steps

Start here right now to build momentum:

- Install 5G-LENA: Follow the install commands in Section 2.2. Run the built-in cttc-nr-demo example to verify everything works.

- Literature survey: Search Google Scholar for the 6 representative papers in Section 6. Download and read the 3 most relevant ones.

- Set up project structure: Create the folder structure from Section 7. Initialize a git repository.

- Read 3GPP TS 38.331: Focus on sections 5.3 (RRC connection control) and 5.4 (inter-RAT mobility). This is the spec your algorithm modifies.

- Ask Claude for the code: Once ns-3 is working, come back and ask for the full C++ baseline simulation script and Python pipeline — these will be generated for you.

This document is your complete research blueprint. Good luck!