DEPARTMENT OF ELECTRONICS & COMMUNICATION ENGINEERING

VIII Sem Major Project Synopsis

“Title”

Submitted by

| Students Name1 USN1 Students Name3 USN3 | Students Name2 USN2 Students Name4 USN4 |
| --- | --- |

Under the Guidance of

| Guide Name and Designation |  |
| --- | --- |

2025- 26

Synopsis must include the following

- Introduction

Fifth Generation New Radio (5G NR) networks are being deployed globally, promising ultra-high data rates and massive connectivity. However, a critical challenge is the rapid battery depletion of User Equipment (UE) caused by inefficient Radio Resource Control (RRC) state management. The 3GPP RRC state machine governs transitions between RRC_IDLE, RRC_INACTIVE, and RRC_CONNECTED states, yet current implementations rely on static inactivity timers that cannot adapt to diverse traffic patterns and UE mobility. This project proposes ARSTA (Adaptive RRC State Transition Algorithm), a novel algorithm that dynamically optimises RRC state decisions using EWMA-based traffic prediction, mobility-aware DRX tuning, and handover-aware state locking to achieve 25–35% energy savings without degrading Quality of Service.

- Literature Survey

Ding et al. [1] (IEEE TMC, 2024) proposed 5GSaver, which exploits RRC_INACTIVE state to eliminate radio tail energy in 5G SA networks, achieving 9.5% energy and 12.4% latency reduction. Their work demonstrates that proactive INACTIVE state entry is highly effective for burst-silence traffic patterns, motivating ARSTA's EWMA-based approach.

Khlass et al. [2] (IEEE VTC, 2019) evaluated flexible RRC configurations in ns-3, finding that optimal INACTIVE+DRX achieves over 200% latency reduction vs. IDLE and 40% power saving vs. CONNECTED under bursty traffic, establishing key benchmark values used in this project.

Hoglund et al. [3] (IEEE Comms. Magazine, 2019) introduced the foundational design of the 5G RRC_INACTIVE state, demonstrating 8× connection setup latency improvement and the benefits of UE context preservation through RNA-based paging.

Lin et al. [4] (IEEE COMST, 2023) provided the most comprehensive DRX survey covering LTE to 6G, identifying mobility-aware adaptive DRX tuning as an open research gap and confirming ML-based DRX achieves 15–35% better energy saving than static 3GPP configurations.

Islam et al. [5] (IEEE JSAC, 2023) studied 5G-Advanced Rel-18 network energy saving techniques, showing 25–40% network energy reduction with AI-assisted gNB sleep and identifying UE-side algorithms as a complementary open problem.

Raghunath et al. [6] (IEEE GLOBECOM, 2022) demonstrated mobility-aware RRM relaxation achieving 64–96% measurement energy saving by adapting DRX cycles to UE speed class, directly motivating ARSTA's velocity-based DRX tuning module.

Nokia Bell Labs [7] (IEEE WCNC, 2023) achieved 93% handover failure prediction accuracy using XGBoost on RSRP and velocity features, establishing that HO events are predictable with sufficient advance time to prevent unnecessary RRC state transitions — the basis for ARSTA's handover-aware state locking.

Ma et al. [8] (Elsevier DCN, 2024) proposed Predictive-PEI using LSTM-FNN for paging arrival prediction, achieving 38.89% paging energy saving while maintaining 99.2% success rate, representing the closest competitor to ARSTA's paging optimisation component.

Quek et al. [9] (IEEE Comms. Letters, 2021) analysed Small Data Transmission (SDT) in RRC_INACTIVE using configured-grant, demonstrating 73% power efficiency and 85% latency reduction for sub-500-byte payloads, informing ARSTA's fast-resume design for short bursts.

Polese et al. [10] (IEEE WNS3, 2021) implemented the 3GPP TR 38.840 energy model in ns-3 for 5G NR UEs, validating within 8% error against commercial measurements, providing the simulation energy accounting infrastructure extended in this project.

- Motivation

Despite advances in 5G NR standards, a comprehensive survey (García-Martínez et al., Sensors 2024) explicitly identifies combining traffic prediction with INACTIVE state exploitation as an open research gap. Current deployments waste significant energy through: (i) static inactivity timers that cause unnecessary CONNECTED-state occupancy for applications with bursty traffic; (ii) absence of mobility-aware DRX configuration, resulting in sub-optimal sleep cycles for high-speed UEs; (iii) no mechanism to prevent costly RRC re-establishment when state transitions coincide with handover windows; and (iv) paging energy wasted on monitoring channels when no pages are imminent. Smartphones consume up to 900 mW in RRC_CONNECTED vs. only 15 mW in RRC_INACTIVE (3GPP TR 38.840), making intelligent state management the most impactful lever for extending UE battery life in 5G networks.

- Problem Statement

Current 5G NR RRC state management employs fixed network-configured inactivity timers (typically 10–30 s) that are traffic-agnostic, velocity-agnostic, and unaware of upcoming handover events. This results in: excessive CONNECTED-state dwell for bursty applications, frequent and energy-wasteful IDLE↔CONNECTED transitions, suboptimal DRX cycle selection for mobile UEs, and RRC re-establishment failures when INACTIVE↔CONNECTED transitions coincide with handovers. No existing algorithm simultaneously optimises all four dimensions — traffic-adaptive timers, mobility-aware DRX, handover-aware state locking, and paging prediction — within a single unified framework.

- Objectives

- Design ARSTA: a unified RRC state optimisation algorithm combining EWMA traffic prediction, velocity-based DRX tuning, handover-aware state locking, and paging area adaptation.

- Achieve 25–35% reduction in total UE energy consumption compared to the 3GPP static-timer baseline.

- Reduce unnecessary RRC state transitions by 40–50% while maintaining handover success rate ≥ 98% and paging success rate ≥ 99%.

- Implement and validate ARSTA in ns-3.44 with the 5G-LENA NR module, extending it with a custom RRC_INACTIVE state and the TR 38.840 energy model.

- Validate results statistically across 10 simulation seeds per configuration with 95% confidence intervals.

- Methodology

ARSTA operates as a UE-side algorithm with four interlocking modules. The Traffic Prediction Module maintains an Exponentially Weighted Moving Average (EWMA) of inter-packet arrival times with adaptive smoothing factor α, estimating the probability of extended idle periods to trigger early INACTIVE entry before the inactivity timer expires. The Mobility-Aware DRX Module reads UE velocity from the mobility model and configures DRX cycle length inversely proportional to speed — stationary UEs use long cycles (160 ms) for maximum energy saving, while high-speed UEs use short cycles (20 ms) to remain responsive during frequent handovers. The Handover-Aware State Locking Module monitors RSRP gradients; when a rapid RSRP decline indicates an impending handover, it locks the UE in CONNECTED state for the predicted handover window duration to avoid INACTIVE→HO re-establishment overhead. The Paging Optimisation Module uses EWMA estimates of paging inter-arrival time to adaptively size the RNA area, reducing paging overhead for stationary UEs while maintaining reachability. The algorithm is validated in ns-3.44 with 3 gNBs arranged in a triangular topology, 20–50 UEs under RandomWaypoint mobility (0–30 m/s), 3.5 GHz FR1 channel at 40 MHz bandwidth, bursty Poisson traffic, and 300 s simulation runs per seed.

- Hardware / Software Tools Used

- ns-3.44 Network Simulator with 5G-LENA NR module (nr-v4.0) — primary simulation platform

- Python 3.11 with NumPy, Pandas, Matplotlib, SciPy — data processing, energy modelling, statistical analysis and publication-quality plotting

- MATLAB R2023b — analytical Markov chain energy model and ns-3 result validation

- Ubuntu 24.04 LTS — development and simulation environment (existing lab hardware)

- 3GPP TS 38.331, TR 38.840, TR 38.901 — normative protocol and energy model specifications

- Budget Estimate

All primary tools are open-source or covered by university licenses.  ns-3 / 5G-LENA Simulator — Open-source (free): ₹0.  Python / NumPy / Matplotlib — Open-source (free): ₹0.  MATLAB R2023b — University license (existing): ₹0.  Computation Hardware — Existing lab workstation: ₹0.  IEEE Xplore / Elsevier — University library access: ₹0.  Printing & Binding — Synopsis + Final Report: ₹500.  Total estimated expenditure: ₹500 (printing and binding only).

- Project Timeline

The project follows a 10-week schedule as shown in the Gantt chart below:

| Activity |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ns-3 Setup & Baseline | ■ | ■ |  |  |  |  |  |  |  |  |
| Energy Model Design |  |  | ■ |  |  |  |  |  |  |  |
| ARSTA Python Prototype |  |  |  | ■ |  |  |  |  |  |  |
| ns-3 C++ Integration |  |  |  |  | ■ | ■ |  |  |  |  |
| Experiments & MATLAB |  |  |  |  |  |  | ■ | ■ |  |  |
| Paper Writing |  |  |  |  |  |  |  |  | ■ |  |
| Review & Submission |  |  |  |  |  |  |  |  |  | ■ |

- References

[1] Z. Ding et al., "Energy Optimization for Mobile Applications by Exploiting 5G Inactive State," IEEE Trans. Mobile Comput., vol. 23, no. 12, 2024. DOI: 10.1109/TMC.2024.3377696

[2] A. Khlass et al., "On the Flexible and Performance-Enhanced RRC for 5G NR Networks," Proc. IEEE VTC2019-Fall, pp. 1–6. DOI: 10.1109/VTCFall.2019.8891551

[3] A. Hoglund et al., "RRC State Handling for 5G," IEEE Commun. Mag., vol. 57, no. 1, pp. 44–49, 2019. DOI: 10.1109/MCOM.2018.1700957

[4] K.-H. Lin et al., "A Survey on DRX Mechanism: Device Power Saving From LTE and 5G NR to 6G," IEEE Commun. Surveys Tuts., vol. 25, no. 1, pp. 156–183, 2023. DOI: 10.1109/COMST.2022.3217854

[5] T. Islam et al., "Enabling Network Power Savings in 5G-Advanced and Beyond," IEEE J. Sel. Areas Commun., vol. 41, no. 7, pp. 1888–1899, 2023. DOI: 10.1109/JSAC.2023.3271241

[6] P. Raghunath et al., "Energy Efficient RRM Relaxation for Reduced Capability UEs in 5G Networks," Proc. IEEE GLOBECOM 2022, pp. 1–6. DOI: 10.1109/GLOBECOM48099.2022.10000873

[7] "ML-Based Handover Failure Prediction Model for HO Success Rate Improvement in 5G," Proc. IEEE WCNC 2023, pp. 1–6. DOI: 10.1109/WCNC55385.2023.10060203

[8] W. Ma et al., "Research on Paging Enhancements for 5G-A Downlink Transmission Energy Saving," Digital Commun. Networks (Elsevier), vol. 11, no. 3, pp. 818–828, 2024. DOI: 10.1016/j.dcan.2024.07.005

[9] T. Q. S. Quek et al., "Efficient Handling of Small Data Transmission for RRC Inactive UEs in 5G," IEEE Commun. Lett., vol. 25, no. 9, pp. 3041–3045, 2021. DOI: 10.1109/LCOMM.2021.3098069

[10] M. Polese et al., "An ns3-based Energy Module of 5G NR UEs for Millimeter Wave Networks," Proc. IEEE WNS3, 2021. DOI: 10.1145/3460797.3460805

[11] 3GPP TS 38.331 V18.3.0, "NR; Radio Resource Control (RRC); Protocol Specification," Rel. 18, 2024.

[12] 3GPP TR 38.840 V16.0.0, "Study on UE Power Saving in NR," Rel. 16, Jun. 2019.

Important Instructions

- Synopsis should not be more than 4 pages

- Please follow the font type of Times New Roman with size 14 for the headings and size 12 for text.

- The complete document should be justified (distribute the text evenly between the margins)

- The document should be printed on both sides of a paper.

- Do not copy paste the PPT slides directly here.

| Status: | Accepted/Not Accepted |
| --- | --- |

| Guide Name: Signature: | Panel Name: Signature: |
| --- | --- |